PRODUCT REQUIREMENTS DOCUMENT

Context-Bound Secure Provisioning Architecture

for Bluetooth Mesh Networks

Subject: Computer Communications and Networks

Version 2.0  |  February 2026

# 1. Executive Summary

This project designs, implements, and evaluates a secure, scalable, and context-aware device provisioning protocol for Bluetooth Mesh networks, framed as a communication protocol engineering project for the subject Computer Communications and Networks.

The project is structured around three core CCN pillars: protocol design and specification, network communication simulation, and performance evaluation. A lightweight ML-based anomaly detection layer is integrated as a behavioral defense mechanism, complementing the cryptographic protocol with network-level threat detection.

The solution combines:

- Protocol design across Bluetooth Mesh network layers

- Device-specific identity secrets and AES-based authentication

- Context-bound transcript verification to prevent relay and misbinding attacks

- Python-based network simulation with packet-level message exchange

- ML anomaly detection (Isolation Forest) for behavioral threat identification

- Performance evaluation using CCN metrics — latency, message overhead, scalability

# 2. Problem Statement

Bluetooth Mesh networks require secure onboarding of new devices before they can participate in the network. Existing provisioning mechanisms face several networking and security limitations:

- Shared bootstrap keys create single-points-of-failure across the network

- Manual PIN/OOB provisioning does not scale to large mesh deployments

- Relay and misbinding attacks exploit the flooding-based mesh communication model

- Compromise of one device can undermine the broader network trust model

From a CCN perspective, the core problem is: how do we design a communication protocol that allows a new node to join a mesh network securely, authenticating itself through a structured message exchange, without relying on shared secrets or manual intervention, and without being vulnerable to network-layer attacks such as replay and relay?

# 3. Goals and Objectives

## 3.1 Primary Goals

- Design a communication protocol for secure device provisioning in Bluetooth Mesh

- Implement the protocol as a Python-based network simulation with real packet exchange

- Prevent replay, relay, and misbinding attacks through protocol-level mechanisms

- Evaluate protocol performance using standard CCN metrics

## 3.2 Secondary Goals

- Integrate ML-based behavioral anomaly detection as a complementary defense layer

- Produce a complete protocol specification document (packet formats, sequence diagrams, state machines)

- Demonstrate layered security combining cryptographic and behavioral defenses

# 4. Scope

## 4.1 In Scope

- Provisioning phase protocol design and specification

- Packet format definition and message exchange implementation

- Python simulation of provisioner and device communication

- Attack simulation (replay, relay, misbinding)

- ML anomaly detection on session-level traffic features

- Performance measurement: latency, message overhead, scalability

## 4.2 Out of Scope

- Hardware-level tamper resistance or firmware development

- Full Bluetooth Mesh stack reimplementation

- Production deployment or real BLE radio communication

- Formal symbolic verification (ProVerif) — replaced with documented threat analysis

# 5. System Overview

## 5.1 Network Topology

The system operates over a simulated Bluetooth Mesh network with the following entities and communication channels:

| Entity | Role | Pre-Provisioning State | Post-Provisioning State |
| --- | --- | --- | --- |
| Provisioner | Trusted network controller (AAA + DHCP equivalent) | Holds device database (DeviceID → K_device) | Assigns unicast address, shares session key |
| Unprovisioned Device | New node requesting network access | Has DeviceID and K_device, no network address | Has assigned unicast address and session key |
| Relay Nodes | Forward provisioning beacon across mesh | Already provisioned mesh members | No change — passive forwarders |
| Adversary | Network-level attacker | Can eavesdrop on provisioning bearer | Attempts replay, relay, misbinding attacks |

## 5.2 Bluetooth Mesh Stack Integration

The provisioning protocol operates at the provisioning bearer layer, below the full mesh network stack. This is the stage before the device is assigned a unicast address and network keys. The stack layers are:

| Layer | Role | Protocol Interaction |
| --- | --- | --- |
| Access Layer | Application-level messaging | Post-provisioning encrypted traffic |
| Upper Transport | End-to-end encryption | Session key usage after provisioning |
| Lower Transport | Segmentation & reassembly | Handles large provisioning packets |
| Network Layer | Addressing and TTL | Unicast address assigned during provisioning |
| Bearer Layer (PB-ADV) | Physical transmission channel | Provisioning protocol runs here |

# 6. Protocol Specification

## 6.1 Protocol State Machines

Provisioner States

IDLE  →  BEACON_RECEIVED  →  CHALLENGE_SENT  →  VERIFYING  →  COMPLETE  /  ERROR

Device States

IDLE  →  BEACON_SENT  →  WAITING_FOR_CHALLENGE  →  RESPONSE_SENT  →  WAITING_FOR_CONFIRMATION  →  PROVISIONED

## 6.2 Message Exchange — Sequence

The protocol uses a 5-message handshake over the provisioning bearer:

| Step | Direction | Message | Purpose |
| --- | --- | --- | --- |
| 1 | Device → Provisioner | Beacon | Device advertises identity and capabilities |
| 2 | Provisioner → Device | Challenge | Provisioner issues fresh nonce for authentication |
| 3 | Device → Provisioner | Response | Device returns context-bound auth token + transcript hash |
| 4 | Provisioner → Device | Confirmation | Provisioner assigns network address, confirms session key |
| 5 | Device → Provisioner | ACK | Device acknowledges — provisioning complete |

## 6.3 Packet Format Definitions

Message 1 — Beacon (Device → Provisioner)

| Field | Size (bytes) | Value / Purpose |
| --- | --- | --- |
| Msg Type | 1 | 0x01 = Beacon |
| DeviceID | 4 | Unique device identifier |
| Capabilities | 2 | Supported algorithm flags |
| NonceD | 16 | Device-generated random freshness value |

Message 2 — Challenge (Provisioner → Device)

| Field | Size (bytes) | Value / Purpose |
| --- | --- | --- |
| Msg Type | 1 | 0x02 = Challenge |
| ProvisionerID | 4 | Provisioner identity — included in transcript for relay prevention |
| NonceP | 16 | Provisioner-generated random — core freshness for replay prevention |
| Timestamp | 8 | Additional replay protection via time window |

Message 3 — Response (Device → Provisioner)

| Field | Size (bytes) | Value / Purpose |
| --- | --- | --- |
| Msg Type | 1 | 0x03 = Response |
| DeviceID | 4 | Echoed for misbinding prevention |
| Auth Token | 16 | AES output binding both identities and full session context |
| Transcript Hash | 32 | SHA-256 of all prior message fields |
| NonceD | 16 | Device nonce — contributes to session key derivation |

Message 4 — Confirmation (Provisioner → Device)

| Field | Size (bytes) | Value / Purpose |
| --- | --- | --- |
| Msg Type | 1 | 0x04 = Confirmation |
| Assigned Address | 2 | Unicast network address assigned to device |
| Session Key ID | 4 | Identifier for the derived session key |
| Confirm Token | 16 | Provisioner's AES proof of successful verification |

Message 5 — ACK (Device → Provisioner)

| Field | Size (bytes) | Value / Purpose |
| --- | --- | --- |
| Msg Type | 1 | 0x05 = ACK |
| DeviceID | 4 | Final identity confirmation |
| Status | 1 | 0x00 = Success, 0x01 = Failure |

## 6.4 Context Binding Formula

The core security mechanism of this protocol is context binding — the authentication token is computed over a transcript that includes all session-relevant identifiers, making it impossible to reuse in a different session or with a different provisioner.

| Operation | Formula | Prevents |
| --- | --- | --- |
| Transcript Assembly | Transcript = DeviceID ¦¦ ProvisionerID ¦¦ NonceP ¦¦ NonceD ¦¦ Timestamp | Misbinding, relay |
| Transcript Hashing | Transcript_Hash = SHA-256(Transcript) | Tampering with any field |
| Auth Token | Auth_Token = AES-128(K_device, Transcript_Hash[0:16]) | Impersonation without K_device |
| Session Key Derivation | Session_Key = AES-128(K_device, NonceP XOR NonceD) | Session key exposure |

# 7. Threat Model & Security Analysis

## 7.1 Adversary Capabilities

- Can eavesdrop on all provisioning bearer traffic

- Can replay previously captured packets

- Can delay, reorder, or relay messages between parties

- Can compromise limited individual devices

- Cannot break AES-128 or invert SHA-256

- Cannot read K_device from the device (assumed secure storage)

## 7.2 Attack Resistance Analysis

| Attack | Network Description | Defeated By | Why It Fails |
| --- | --- | --- | --- |
| Replay | Adversary captures Message 3 and resends in a future session | Fresh NonceP per session | Auth Token bound to old NonceP — mismatch on provisioner verification |
| Relay | Adversary forwards messages between victim device and fake provisioner | ProvisionerID in Transcript | Device binds Auth Token to real ProvisionerID — relay with fake ID produces wrong token |
| Misbinding | Adversary tries to bind a legitimate response to a different device | DeviceID in Transcript + provisioner DB lookup | Provisioner verifies DeviceID matches database entry |
| Eavesdropping | Adversary captures all 5 messages | K_device never transmitted | Session key requires K_device — cannot be derived from observed traffic |
| Impersonation | Adversary pretends to be a legitimate device | Auth Token requires K_device | Without K_device, adversary cannot produce a valid Auth Token |

# 8. ML-Based Behavioral Anomaly Detection

## 8.1 Motivation

Cryptographic verification detects attacks at the message level — it rejects invalid tokens. However, certain attack patterns are visible at the network behavior level before any cryptographic check occurs. An ML anomaly detector acts as a pre-filter, flagging suspicious sessions based on communication timing and frequency patterns.

This creates a layered defense architecture:

- Layer 1 — Protocol level: Context binding and nonce verification (cryptographic)

- Layer 2 — Network level: Packet format validation and transcript verification

- Layer 3 — Behavioral level: ML anomaly detection on session timing patterns

## 8.2 Feature Extraction

| Feature | Description | Attack Signature |
| --- | --- | --- |
| Beacon to Challenge delay (ms) | Time between Message 1 and Message 2 | Relay attack increases this significantly |
| Challenge to Response delay (ms) | Time between Message 2 and Message 3 | Relay attack increases this significantly |
| Device seen before flag | Whether DeviceID appeared in recent sessions | Replay attack reuses same DeviceID |
| Session frequency | Number of provisioning attempts per minute | Brute force / DoS shows high frequency |
| Packet size variance | Deviation from expected packet sizes | Tampered or malformed packets |

## 8.3 Model

Algorithm: Isolation Forest (sklearn) — chosen because it is trained only on normal sessions (unsupervised), requires no labeled attack data, and is computationally lightweight for an IoT network context.

| Parameter | Value | Justification |
| --- | --- | --- |
| Algorithm | Isolation Forest | Unsupervised, no labeled attack data needed |
| Contamination | 0.1 | Assumes up to 10% of training data may be anomalous |
| Training data | Simulated normal provisioning sessions | Generated from main.py simulation runs |
| Test data | Attack simulation sessions from attacker.py | Replay and relay sessions with distinct timing |

# 9. Implementation Architecture

## 9.1 Project File Structure

provisioning_protocol/

- common/crypto.py — AES encryption, SHA-256 hashing, nonce generation, session key derivation

- common/packets.py — Packet construction and parsing for all 5 messages using struct

- common/constants.py — Message type codes, field sizes, status codes

- provisioner.py — Provisioner entity: beacon handling, verification, confirmation

- device.py — Device entity: beacon broadcast, challenge response, session key derivation

- channel.py — Simulated network channel with eavesdropping and delay support

- attacker.py — Attack simulations: replay attack, relay attack, misbinding attempt

- anomaly_detector.py — Isolation Forest model: feature extraction, training, prediction

- main.py — Orchestrates full protocol run + attack simulations + ML evaluation

- tests/ — Unit tests for crypto operations, packet parsing, attack detection

## 9.2 Technology Stack

| Component | Technology | Purpose |
| --- | --- | --- |
| Protocol Implementation | Python 3.10+ | Core language for all simulation logic |
| Cryptography | PyCryptodome | AES-128 and SHA-256 operations |
| Packet Construction | Python struct module | Binary packet serialization/deserialization |
| Network Simulation | Python sockets / in-process channel | Simulated provisioning bearer |
| ML Anomaly Detection | scikit-learn (IsolationForest) | Behavioral anomaly detection |
| Code Editor | VS Code | Development environment |
| Diagramming | draw.io + sequencediagram.org | Architecture and sequence diagrams |
| Documentation | Microsoft Word / Google Docs | Phase reports and protocol spec |

# 10. Project Phases & Roadmap

## Phase 1 — Network Architecture & Protocol Design (Weeks 1–4)

Objective: Produce a complete protocol specification document — the blueprint for implementation.

| Week | Focus | Key Deliverables | Tools |
| --- | --- | --- | --- |
| Week 1 | Bluetooth Mesh background study | Network description document, stack layer diagram | Google Docs, YouTube BT SIG resources |
| Week 2 | Topology and entity roles | Topology diagram, entity role table, addressing scheme | draw.io |
| Week 3 | Protocol specification | 5 packet format tables, state machines, sequence diagram, context binding formula | draw.io, sequencediagram.org, Docs |
| Week 4 | Threat modeling and architecture | Threat analysis table, final architecture diagram, consolidated Phase 1 spec | draw.io, Docs |

## Phase 2 — Protocol Implementation & Simulation (Weeks 5–8)

Objective: Build working Python simulation of the full 5-message handshake with visible packet exchange and attack resistance.

| Week | Focus | Key Deliverables | Tools |
| --- | --- | --- | --- |
| Week 5 | Environment setup and project structure | constants.py, crypto.py, project folder structure | Python, PyCryptodome, VS Code |
| Week 6 | Core modules | packets.py with all 5 message builders/parsers | Python struct, VS Code |
| Week 7 | Entity implementation | provisioner.py, device.py, channel.py — full handshake runs end to end | Python |
| Week 8 | Attack simulation | attacker.py, main.py — replay and relay attacks attempted and defeated | Python |

## Phase 3 — Performance Analysis & ML Integration (Weeks 9–12)

Objective: Evaluate protocol performance using CCN metrics and integrate ML anomaly detection.

| Week | Focus | Key Deliverables | Tools |
| --- | --- | --- | --- |
| Week 9 | Session timing instrumentation | Timing logs added to channel.py, session metadata collection | Python, logging module |
| Week 10 | ML anomaly detector | anomaly_detector.py — Isolation Forest trained on normal sessions, tested on attack sessions | scikit-learn |
| Week 11 | Performance evaluation | Latency measurements, message size analysis, scalability tests with multiple devices | Python, matplotlib |
| Week 12 | Final report and results | Complete evaluation report with tables, graphs, attack detection accuracy | Word/Docs, matplotlib |

# 11. Functional Requirements

| ID | Requirement | Phase |
| --- | --- | --- |
| FR-1 | The system shall define a 5-message provisioning protocol with explicit packet formats for each message | 1 |
| FR-2 | The system shall use fresh nonces per session to ensure replay resistance | 1, 2 |
| FR-3 | Authentication responses must bind DeviceID, ProvisionerID, and session transcript hash | 1, 2 |
| FR-4 | Both provisioner and device shall independently derive the same session key after successful provisioning | 2 |
| FR-5 | The channel simulation shall support message interception for attack scenario testing | 2 |
| FR-6 | The attacker module shall simulate replay and relay attacks with visible outcomes | 2 |
| FR-7 | The ML module shall extract timing features from sessions and classify normal vs anomalous | 3 |
| FR-8 | Performance metrics (latency, message size, session count) shall be measured and reported | 3 |

# 12. Non-Functional Requirements

| ID | Requirement | Target |
| --- | --- | --- |
| NFR-1 | Protocol must be implementable on resource-constrained devices | AES-128 only, minimal message sizes |
| NFR-2 | Provisioning latency overhead must remain acceptable | < 500ms per session in simulation |
| NFR-3 | Compromise of one device must not affect other devices | Device-specific keys, no shared secrets |
| NFR-4 | System must support multiple concurrent provisioning sessions | Tested up to 50 devices in simulation |
| NFR-5 | ML model inference must be fast enough for real-time use | < 10ms per session classification |

# 13. Evaluation Metrics

All metrics are measured in the Python simulation environment and reported in the Phase 3 final report.

| Metric | How Measured | CCN Relevance |
| --- | --- | --- |
| End-to-end provisioning latency | Timestamp delta from Beacon to ACK | Protocol efficiency — round trip time |
| Total bytes per provisioning session | Sum of all 5 packet sizes | Network overhead — bandwidth usage |
| Number of message round trips | Count of message exchanges (fixed at 3 round trips) | Protocol efficiency |
| Scalability — sessions per second | Provision N devices, measure total time | Network throughput |
| Replay attack detection rate | % of replayed packets rejected by protocol | Security effectiveness |
| ML anomaly detection accuracy | % of attack sessions correctly flagged | Behavioral detection effectiveness |
| ML false positive rate | % of normal sessions incorrectly flagged | Operational reliability |

# 14. Success Criteria

The project is considered successful if all of the following are demonstrated:

- The full 5-message handshake completes successfully in simulation with matching session keys on both sides

- Replay attack is visibly defeated — replayed Response packets are rejected due to nonce mismatch

- Relay attack is visibly defeated — transcript hash mismatch detected when ProvisionerID is altered

- Misbinding attack fails — DeviceID field mismatch detected during provisioner verification

- Session keys derived independently by both parties are identical

- ML anomaly detector achieves > 85% accuracy distinguishing normal from attack sessions

- Performance metrics are within acceptable IoT limits (< 500ms latency, reasonable packet sizes)

# 15. Risks and Mitigation

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Simulation too simplistic to demonstrate networking concepts | Medium | High | Use Python sockets for channel simulation to add realism; log and visualize packet exchanges clearly |
| ML model overfits to simulation data | Medium | Medium | Generate varied normal and attack sessions with randomized timing; use cross-validation |
| Performance evaluation lacks comparison baseline | Low | Medium | Compare against standard Bluetooth Mesh provisioning message count and documented latency |
| Protocol specification gaps discovered during implementation | Low | High | Phase 1 produces complete spec before any coding; implementation strictly follows spec |
| Scope creep from attempting ProVerif or hardware integration | Low | High | Explicitly out of scope; focus on Python simulation and ML layer only |

# 16. Future Extensions

- Integration with other mesh protocols such as Zigbee and Thread

- Certificate-based provisioning using asymmetric cryptography

- Hardware root-of-trust support for real BLE devices

- Lightweight post-quantum cryptographic primitives for future-proofing

- Group provisioning protocol for simultaneous multi-device onboarding

- Deep learning anomaly detection (LSTM) for more complex traffic pattern analysis

# Summary

This PRD defines a Computer Communications and Networks course project that designs, implements, and evaluates a secure provisioning protocol for Bluetooth Mesh networks. The project is structured around three phases directly aligned with CCN subject matter: protocol design and specification, network communication simulation with attack scenarios, and performance evaluation augmented with ML-based behavioral anomaly detection.

The project demonstrates mastery of network protocol design (packet formats, state machines, sequence diagrams), communication simulation (entity-to-entity message exchange in Python), network security (replay, relay, misbinding attack resistance), and quantitative performance analysis (latency, overhead, scalability) — all core competencies of the Computer Communications and Networks subject.